# Stream Formats

| Feature/ Observation | RTCM3.0 | RTCM3.0 (Magellan) | SOC | BINEX 0x7f-03 | BINEX 0x7f-05 | RINEX 2.11 | RINEX v3 | Requirements (RT AC) N = Nice to have M = Must |  | Requirements (User Group 2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Status | available | proposed, high correlated with RTCM and RINEX | available | available | Proposed Preliminary draft available | available | available |  |  |  |
| Time Tag Resolution | 1ms | 1ms | 1ms | 1 ms | 1ms | 100ns | 100ns | 100 ns | ## N |  |
| Rec Clock O/S | No (Clock Steering) | No (clock steering) | No | Yes (ns) | Yes (ns) | No |  |  | N |  |
| Rec. Clock Reset | No (Clock Steering) | No (clock steering) | No | Yes +/- 1 ms | Yes +/- 1 ms | No |  |  | N |  |
| Clock Steering Flag | Always steered |  | Always Steered | Yes | Yes | No | No |  | N |  |
| Epoch Independent | yes | Yes | yes | yes | yes | yes |  |  | M |  |
| Pseudorange (PR) | Ca+ P2(Y) or P1(Y) + P2(Y) | Any combination | CA + P1(Y) +P2(Y) | CA or P1(Y), P2(Y) or L2C | All GNSS observations and constellations as defined by RINEX 3 specification | Ca, P1, P2 and L2C | All possible pseudoranges, distinctions between tracking modes | Flexibly support all GNSS observations | M |  |
| PR Resolution | 0.02m | 0.02m | 0.001m | 0.001m | 0.001m | 0.001m | scalable | 0.02 m or better | M |  |
| PR Smoothed Flag | Smoothing Interval |  | No | No | Yes |  |  | Preferably Non-Smoothed | N |  |
| Smoothed + Non-Smoothed PR | No |  | No | No | No | No | No |  | N |  |
| Carrier Phase(CP) | LA+L2 or L1+L2 | Any combination | L1 + L2 | L1 + L2 | Capable of all | L1+L2 | All possible phase obs, distinctions between tracking modes | Flexibly support all GNSS observations | M |  |
| Carrier Phase Resolution | 0.5mm | 1 mm/s, can be done finer | 0.02mm | 0.0001cycle or 0.02mm | 0.02mm | 0.2mm | scalable | 0.5 mm or better | M |  |
| CP Smoothed Flag | No |  | No | No | Yes |  |  | Preferably Non-Smoothed | N |  |
| Multipath Reduction Flag | No |  | No | No | Yes |  |  |  | N |  |
| Doppler | No | Yes | No | Yes | Yes | Yes | All possible doppler obs, distinctions between tracking modes |  | N |  |
| Doppler Resolution | N/A | 1 mm/s, can be done finer | N/A | 1/256 Hz | 1/256 Hz | ? | scalable |  |  |  |
| CNo | L1+L2 | Any | CA, P1,P2 | L1(CA or P1) and L2 (P2) | All codes have an SNR field | S1, S2 | Signal strengths for all carriers, distinctions between tracking modes |  | M |  |
| Cno Resolution | 0.25 dbHz | 0.25 dBhz, can be done worse to save the size | 1 db Hz | 0.4 or 0.1 db Hz | 0.4 or 0.1 db Hz | Rx Dependent | scalable |  |  |  |
| Cycle Slip Flag |  | No | No | Yes | Yes |  | yes |  |  |  |
| Loss of Lock Counter | Lock Time | Lock time | Lock Period | Slip count | Slip Count | Slip Flag | Slip flag | Any technique that can be used to derive Loss of Lock indication | M |  |
| SV Health Flag | In ephemeris message | In ephemeris message | Yes | Yes | Yes | Health info in nav mess files | Health info in nav mess files | Nav message | M |  |
| Generic GNSS Observation Blocks |  | Any subset of pseudorange carrier CNR Doppler | No | No | Yes : obs type, code, phase SNR Doppler, slip count |  |  |  | N |  |
| Glonass Support | Yes | Yes, in similar message | No | Yes | Yes | Yes | Yes |  | M |  |
| Designed to support future GNSS observations | Yes (Initial Galileo Messages ready) | Yes | No | Yes | Yes, Uses RINEX 3.0 specifications | ? | Yes |  | M |  |
| Open Format | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | M |  |
| Documentation Cost | US$ 80 | Draft | free | free | free | free | free |  |  |  |
| Documentation can be published on a public web site | No | No | Yes | Yes | Yes | Yes | Yes | In keeping with IGS policies of open data and product formats. | M |  |
| Quality of documentation | High | High | Limited | Good | Good | Good | High |  | N |  |
| Open Source Code available | Decode, Encode: irrelevant, comes with firmware | Decode, Encode: irrelevant, comes with firmware | Yes(Encode/Decode) | Yes(Encode/Decode) | Yes(Encode/Decode) | Yes | Yes |  | N |  |
| Data compression | Yes – High rate compression | Yes, by options | Yes | Yes | Yes | Yes a separate utility | Separate utilities |  | M |  |
| Compression Performance | Example: 275 bytes for 13 GPS + 4 GLO Satellites | Same as in existing RTCM3 messages | 12 bytes + 21 bytes per SV | 12 bytes + 24 bytes per SV(Code,Phase,SNR, LLI and Doppler for two frequencies) | 12 bytes + 38 bytes per SV(Code,Phase,SNR, LLI, Slip Count and Doppler for two frequencies) |  |  |  |  |  |
| Station ID | Yes | Yes | Requires a wrapper | Requires a BINEX 0x00 rec. | Requires a BINEX 0x00 rec. | Yes | Name, number, type |  | M |  |
| TEQC Support | No | No | Yes | Yes | Yes Planned | Yes | Big question | Should not drive the format decision. | N |  |
| NRIP Support | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | N |  |
| BNC Support | Yes | Yes | Yes | No | Not Planned | Yes | Yes |  | N |  |
| UDPRelay Support | Yes | Yes | Yes | Yes | Yes | No | No |  | N |  |
| UDPRelay Client Support | No | No | Yes | No | No | No | No |  | N |  |
| CRC Authentication | CRC Checksum (24 Bits) | CRC Checksum (24 Bits) | UDP Transport level | Yes | Yes | No | No |  | M |  |
| Supported by receiver firmware | Yes, all geodetic receivers | Yes, all geodetic receivers | No | NetRS, NetR5, GRX1200 series, MicroZ CGRS, geod. Topcon receivers, all receivers stream out only | Planned | Yes | Status currently unknown |  | N |  |
| Secured interoperability of implementations | Yes | Yes | Limited | Yes | Yes | Yes |  |  | M |  |
| International adopted industrial standard | Yes | Yes | No | Limited | Limited | Yes |  |  | N |  |
| Broadcast Ephemeris GPS+GLO | Yes | Use existing RTCM3 ephemeris messages | Yes | 0x01-00 and 0x01-01 For GPS, Glonass being developed | 0x01-00 and 0x01-01 For GPS, Glonass being developed | Yes | Yes |  | M |  |
| Multiplexed Stream (multiple stations within one stream) | Yes | Yes | Yes | Yes | Yes | N/A | N/A |  | M |  |
| Rx-Type, FW Version, Ser-no. | Yes | Already exist in RTCM3 | No | Yes - 0x00 record type | Yes- 0x00 record type | Yes | Yes |  | M |  |
| Antenna Type, Ser-No., Setup-ID, Ant.-Height | Yes | Already exist in RTCM3 | Relies on IGS Log file | Yes- 0x00 | Yes- 0x00 | Yes | Yes |  | M |  |
| Meteorological data | No | No | Yes | Yes 0x7e-00 | Yes 0x7e-00 | Yes | Yes |  | M |  |
| frequency lock to external ref.-frequency indicator | No | No | No | No | No - Can be added as a Obs Flag | No | No |  | N |  |
| RX internal temperature | No | No | No | 0x7d-00 Record | 0x7d-00 Record | No | No |  | N |  |
| Tilt sensor data | No | No | No | No | No | No | No |  | N |  |
| Encryption | Under interoperability testing |  | Transport protocol Level | Transport protocol Level | Transport protocol Level | N/A | N/A |  | N |  |
| Static/Kinematic mode | No | No | No | No | No - Can be added as a Obs Flag | Yes | Yes |  | N |  |

Additional comments regarding RTCM 3.0 (Magellan) proposal:

- Similarly with GPS(+SBAS) message, any GNSS message can be built. If needed, Magellan is ready to provide similar proposal for GLONASS and Galileo.

- The format can be easy modified to meet any resolution/range requirements, because no field is derived against any other.

It is supposed that RTCM-3 will continue with standardization new messages and currently absent Messages (e.g. SBAS ephemeris, meteo data) can be standardized as soon as required by RTCM

Transport Protocols

| Feature | RTCM Ntrip v1 | RTCM Ntrip v2 | UDPRelay | Requirements (RT AC) N = Nice to have M = Must |  |
| --- | --- | --- | --- | --- | --- |
| Status | available | Proposed, implementations available | available |  |  |
| Open Format | Yes | Yes | Yes |  | M |
| Open Source Code available | Yes | Yes | Yes |  | M |
| Quality of documentation | High | High | Good |  | M |
| Documentation cost | US$ 50 |  | Free |  |  |
| Documentation can be published on a public web site | Protocol details: No General description and source-table setup: Yes | Protocol details: No General description and source-table setup: Yes | Yes | In keeping with IGS conventions all data and product formats must be public. | M |
| TEQC Support | No | No | No | Should not drive the protocol decision | N |
| BNC Support | Yes | Planned | No |  | N |
| UDPRelay Client | No | No | Yes |  | N |
| Supported by receiver firmware | Yes | Planned | No |  |  |
| Secured interoperability of implementations | Yes | Once RTCM standard: Yes | Yes With in RTIGS working group |  | M |
| International adopted industrial standard | Yes | Once RTCM standard: Yes | No |  | N |
| Internet Protocol Support | Yes | Yes | Yes |  | M |
| Fully HTTP compatible, can be optional | Client-Caster: Yes Server-Caster: No | Yes | No |  | N |
| TCP | Yes | Yes, optional | No |  | N |
| UDP | No | Yes, optional | UDP |  | N |
| Independent from stream format | Yes | Yes | Yes |  | M |
| Meta-data for site/receiver/format | Yes, source-table | Yes, source-table | Yes Station ID |  | M |
| Bandwidth limits of implementations per stream | Min: ~0.05 kbits, Max: ~30 kbits | Min: ~0.05 kbits, Max: ~30 kbits | No But maximum number of users can be specified |  |  |
| Stream protection, client authorization/authentication | Yes | Yes | Yes |  | M |
| Persistent client access control | Yes | Yes | Yes |  | N |
| Data Authentication |  |  | Yes |  | N |
| Data Encryption |  |  | Yes |  | N |
| Qualified for mass usage | Yes | Yes | Yes |  | N |