# Leica DNA03 Digital Level Settings

Firmware Version 200.4530      Revision Date: 7/26/07

# The following format is used to identify instrument options throughout this reference:

# [Keyboard Key] - Brackets surround selection found on the keyboard.

# “Display Title” - Quotations surround titles found in the instrument display

# <Display Action> - Bracketed greater than and less than symbols surround action options on the display.

# #  Menu Option – Underlines for number and listed menu item found on display.

Item:        Fill-In – Bold indicates required fill-in information.  Italicized bold indicates variable input project/equipment/section specific and regular bold indicates standard fill-in that typically does not change.

Turn the instrument on and the “MEAS & REC” screen appears.

#### 1)  Set Units

From the “MEAS & REC” screen:	Press the [SHIFT][PROG] keys.

From the “MENU” screen, select 2  ALL SETTINGS then press the [enter] key.

From the “ALL SETTINGS” screen, select 4  UNITS, then press the [enter] key.

From the “SET UNITS” screen, set the following:

Distance  :	metre [↔]

Temp. 	    :	     ° C [↔]

Highlight <SET>, then press the [enter] key.

## 2)  Set Date and Time

From the “MEAS & REC” screen:	Press the [SHIFT][PROG] keys.

From the “MENU” screen, select 2  ALL SETTINGS, then press the [enter] key.

From the “ALL SETTINGS” screen, select 5  DATE / TIME, then press the [enter] key.

From the “SET DATE / TIME” screen, set the following:

Date		:	dd.mm.yyyy

Time (24HR)	:	hh:mm:ss

Highlight <SET>, then press the [enter] key.

# 3)  Set Curvature Correction Off

From the “MEAS & REC” screen:	Press the [SHIFT][PROG] keys.

From the “MENU” screen, select 2  ALL SETTINGS, then press the [enter] key.

From the “ALL SETTINGS” screen, select 2  MEASURING, then press the [enter] key.

From the “MEASURING SETTINGS” screen, set the following:

CodeSet	:	     Before [↔]

Decimals	:	0.00001m [↔]

GSI-Format	:	    GSI – 8 [↔]

EarthCurv	:	            No [↔]

Highlight <SET>, then press the [enter] key.

# 4)  Set Increment Mode

From the “MEAS & REC” screen:	Press the [SHIFT][USER] keys.

From the “FUNCTIONS” screen, select 4  PtID & INCREMENT, then press the <enter> key.

From the “PtID & INCREMENT” screen, set the following:

Running PtID

PTID:		1

INCR:		1

Highlight <SET>, then press the [enter] key.

# 5)  Set Measurement Mode

From the “MEAS & REC” screen:	Press the [MODE] key.

From the “MEASURE MODE” screen, set the following to:

Mode		:	     Mean s [↔]

n		:	leave blank

n min		:	               3

n max		:	             10

sDevM / 20m	:	0.00002 m

Highlight <SET>, then press the [enter] key.

#### 6)  Assign “Code” Function to the User Key

From the “MEAS & REC” screen:	Press the [SHIFT][PROG] keys.

From the “MENU” screen, select 1  QUICK SETTINGS then press the [enter] key.

From the “QUICK SETTINGS” screen, toggle “USER-Key” to “Code” like the following:

Contrast  	:	         50% [↔]

EarthCurv	:	           No [↔]

USER-Key	:	       Code [↔]

Decimals	:	0.00001m [↔]

Highlight <SET>, then press the [enter] key.